# Assistive Backpack V1
Library User Guide

ESI 2026 • Core architecture, enums, tasks, inventory, events, and integration

Purpose: help a new developer understand the hardware-independent C++ core and use it as the foundation for an assistive backpack that tracks tagged items, checks task requirements, and communicates events to a feedback layer.

# 1. Project Overview

The core is designed to manage inventory, preset tasks, backpack state, and a queue of events. RFID hardware detects tags; the core maps tag IDs to item categories and maintains item location; the feedback layer consumes events and turns them into audio, LED, or display output. Keep hardware-specific APIs (such as Arduino Serial, RC522 drivers, and audio/display libraries) outside the shared core.

The latest project save point reports that the core is packaged as an Arduino library, the BasicBackpack sketch compiled for Arduino Uno, and desktop CMake builds use the same shared core source. The eventual target is Arduino Mega. RFID and feedback hardware integration still requires testing.

# 2. Architecture at a Glance

| Component | Responsibility | How it connects |
| --- | --- | --- |
| Backpack | Coordinates the overall state machine and task workflow. | Public interface for setup, task control, scan/add/remove hooks, and event retrieval. |
| Inventory | Stores registered item records and their locations. | Maps RFID tag IDs to ItemType and tracks whether each item is outside or inside. |
| TaskManager | Stores preset tasks and tracks the active task. | Checks task requirements against the Inventory instance supplied to it. |
| EventGenerator | Queues BackpackEvent values. | Provides events to the application/feedback layer. |
| RFID adapter | Detects tag IDs from the physical reader. | Calls agreed Backpack hooks; a scan alone may not reveal entry versus removal. |
| Feedback adapter | Produces audio, LED, or display feedback. | Consumes events and interprets event type, item, and task. |
| Desktop / Arduino console | Provides a human-operated test interface. | Uses desktop terminal I/O or Arduino Serial, outside the shared core. |

# 3. Configuration Limits

The supplied Config.h defines fixed capacities to keep memory use predictable on embedded hardware.
These values can be updated in the Config.h file in the \src of the Arduino library.

| Constant | Value | Meaning |
| --- | --- | --- |
| MAX_TASK_ITEMS | 10 | Maximum item entries stored in one Task. |
| MAX_ITEMS | 20 | Maximum item records stored by Inventory. |
| MAX_TASKS | 5 | Maximum registered tasks stored by TaskManager. |
| MAX_EVENTS | 10 | Maximum events stored in EventGenerator's fixed queue. |
| MAX_RFID_UID_BYTES | 10 | Maximum number of bytes read from an RFID tag |

# 4. Enum Reference — Available Values

The names below are taken from the supplied headers. Use scoped names such as ItemType::Phone in C++. Comments in the headers show the intended integer values, but code should normally use the enum names.

## 4.1 ItemType — item category

ItemType represents what the item is. It is not the RFID tag ID and does not describe location. 
These values can be updated in the Item.h file in the \src of the Arduino library.

| Enumerator | Value | Meaning |
| --- | --- | --- |
| ItemType::Unknown | 0 | Item category is unknown or not yet assigned. |
| ItemType::Wallet | 1 | Wallet. |
| ItemType::Laptop | 2 | Laptop. |
| ItemType::Phone | 3 | Phone. |
| ItemType::Keys | 4 | Keys. |
| ItemType::Charger | 5 | Charger. |
| ItemType::WaterBottle | 6 | Water bottle. |
| ItemType::Medication | 7 | Medicine. |
| ItemType::Glasses | 8 | Glasses. |
| ItemType::Umbrella | 9 | Umbrella. |
| ItemType::Book | 10 | Book. |
| ItemType::Headphones | 11 | Headphones. |
| ItemType::Diary | 12 | Diary. |

## 4.2 Location — item location

Location represents where the item is considered to be.

| Enumerator | Value | Meaning |
| --- | --- | --- |
| Location::Unknown | 0 | Location is not known. |
| Location::Outside | 1 | Item is outside the backpack. |
| Location::Backpack | 2 | Item is inside the backpack. |

## 4.3 TaskType — preset task category

TaskType identifies which registered task to start. The enum name alone does not define the task's item list.
These values can be updated in the Task.h file in the \src of the Arduino library.

| Enumerator | Value | Meaning |
| --- | --- | --- |
| TaskType::None | 0 | No active task / sentinel value. |
| TaskType::University | 1 | University task. |
| TaskType::Gym | 2 | Gym task. |
| TaskType::Work | 3 | Work task. |
| TaskType::Doctor | 4 | Doctors’ appointment. |
| TaskType::Park | 5 | Park excursion task. |
| TaskType::FriendsHouse | 6 | Visit a friends house. |

## 4.4 BackpackState — state-machine state

These are the four states declared in Backpack.h. The exact transition rules should be confirmed in Backpack.cpp.

| Enumerator | Value | Meaning |
| --- | --- | --- |
| BackpackState::IDLE | — | No active workflow is being run. |
| BackpackState::CHECKING_INVENTORY | — | Checking whether the active task's required items are present. |
| BackpackState::ACQUIRING_ITEMS | — | One or more required items need to be brought into the backpack. |
| BackpackState::TASK_REMINDER | — | Task reminder/ready state. |

## 4.5 EventType — event vocabulary

EventType identifies what happened. A BackpackEvent also carries an ItemType and a TaskType for context. Interpret those fields according to the event type; the supplied header does not document which fields are unused for each event.

| Enumerator | Value | Meaning |
| --- | --- | --- |
| EventType::Unknown | — | Generic or unspecified event. |
| EventType::TaskStarted | — | A task has started. |
| EventType::TaskStopped | — | A task has stopped. |
| EventType::TaskComplete | — | A task has completed. |
| EventType::ItemRequired | — | An item is required for the task. |
| EventType::ItemAcquired | — | A required item has been acquired for the active task. |
| EventType::ItemAlreadyPresent | — | An item was already present; confirm the exact trigger in implementation. |
| EventType::ItemNotRequired | — | An item is not required for the current task. |
| EventType::UnknownRFID | — | An RFID tag was not recognised or registered. |
| EventType::ItemAdded | — | An item was added to the backpack. |
| EventType::ItemRemoved | — | An item was removed from the backpack. |
| EventType::TaskReminder | — | A task reminder should be presented. |

# 5. Data Structures

## 5.1 Item

Declared in Item.h:

RFIDTagID — RFID tag identifier for the physical item.

ItemType type — category of item, such as ItemType::Phone.

Location location — current recorded location, such as Location::Backpack.

A tag ID identifies a registered physical tag; ItemType describes the category assigned to that tag. Do not use a tag ID where a Task requires an ItemType. The RFIDTagID is stored as a variable length byte array (maximum 10 bytes). The most common length is 4 bytes.

| - struct RFIDTagID - { - uint8_t bytes[MAX_RFID_UID_BYTES]; - uint8_t length; - }; | - struct Item - { - RFIDTagID tagID; - ItemType type; - Location location; - }; |
| --- | --- |

## 5.2 Task

Declared in Task.h:

TaskType type — task category (University, Gym, or Work; None is the sentinel).

ItemType requiredItems[MAX_TASK_ITEMS] — fixed-size array of required item categories.

uint8_t itemCount — number of valid entries in requiredItems.

uint8_t reminderAudio — reminder-audio identifier/value; the supplied header does not define the mapping to audio files.

| - struct Task - { - TaskType type; - ItemType requiredItems[MAX_TASK_ITEMS]; - uint8_t itemCount; - uint8_t reminderAudio; - }; |  |
| --- | --- |

A task's actual requirements are configured in its Task object. For example, a University task could require Laptop, Phone, and Wallet, but those requirements are not encoded by TaskType::University itself.

## 5.3 BackpackEvent

Declared in Events.h:

EventType type — event that occurred.

ItemType item — related item category.

TaskType task — related task category.

This structure is the connection point between the core and the feedback layer. The feedback layer can map event types to prompts, audio, LEDs, or display messages.

| struct BackpackEvent { EventType type; ItemType item; TaskType task; }; |  |
| --- | --- |

# 6. Class APIs and Function Guide

## 6.1 Backpack

Backpack coordinates inventory and task behaviour. Its constructor takes an Inventory reference, and the class contains a TaskManager and EventGenerator. The Inventory object supplied to Backpack must remain alive for as long as Backpack uses it.

| Method | Purpose / notes |
| --- | --- |
| Backpack(Inventory& inventory) | Constructs a Backpack around the supplied Inventory instance. |
| begin() | Initialisation hook for the core. |
| getState() const | Returns the current BackpackState. |
| registerTask(const Task& task) | Registers a task through Backpack's private TaskManager. |
| startTask(TaskType type) | Attempts to start a registered task; returns bool to indicate success/failure. |
| stopTask() | Stops the active task. |
| itemScanned(RFIDTagID tagID) | Hook for an RFID scan. Exact edge-case behaviour must be checked in Backpack.cpp. |
| itemAdded(RFIDTagID tagID) | Reports that the registered item entered the backpack. |
| itemRemoved(RFIDTagID tagID) | Reports that the registered item left the backpack. |
| getCurrentTask() const | Returns the current TaskType. |
| getNextRequiredItem() const | Returns the next required ItemType as defined by TaskManager. |
| isTaskActive() const | Reports whether a task is active. |
| isTaskComplete() const | Reports whether the active task is complete. |
| isItemRequired(ItemType type) const | Checks whether an item category is required by the active task. |
| hasEvent() const | Checks whether an event is waiting in the event queue. |
| getEvent() | Retrieves an event from the event queue; check EventGenerator.cpp for empty-queue behaviour. |
| clearEvents() | Clears queued events. |

## 6.2 Inventory

Inventory stores a fixed-size array of Item records. Based on the supplied Inventory.h, its public methods are:

| Method | Purpose / notes |
| --- | --- |
| clearInventory() | Clears the inventory records. |
| getItemCount() const | Returns the current number of item records. |
| findItem(RFIDTagID tagID) | Finds an item by tag ID and returns Item*; inspect Inventory.cpp for the not-found result. |
| registerItem(RFIDTagID tagID, ItemType type) | Registers a tag ID and the item category; returns bool. |
| addItem(RFIDTagID tagID) | Adds a registered tag to the backpack; returns bool. |
| removeItem(RFIDTagID tagID) | Removes a registered tag from the backpack; returns bool. |
| hasItem(ItemType type) const | Checks whether the item category is present. |
| getItemType(RFIDTagID tagID) const | Returns the category associated with the tag ID; inspect implementation for unknown-tag behaviour. |

The latest project save point also records a new accessor `Item* getItemAt(uint8_t index)`, returning nullptr when index is out of range. This accessor is useful for safe iteration by a console or display layer. Check the repository's current Inventory.h before coding because the pasted header and save point differ.

## 6.3 TaskManager

TaskManager(Inventory& inventory) — constructed with the Inventory it should consult.

registerTask(const Task& task) — stores a task if accepted by the implementation.

startTask(TaskType type) — attempts to activate a registered task.

stopTask() — stops the active task.

getCurrentTask() const — returns the active TaskType.

getNextRequiredItem() const — reports the next item category still required.

isTaskComplete() const — reports completion status.

isTaskActive() const — reports whether a task is active.

isItemRequired(ItemType type) const — checks the active task's required list.

The supplied design notes state that only one task is active at a time and that TaskManager is private inside Backpack. Application code should therefore register/start tasks through Backpack rather than reaching into TaskManager directly.

## 6.4 EventGenerator

emit(const BackpackEvent& event) — attempts to enqueue an event; returns bool, so handle possible queue-full failure.

hasEvent() const — checks whether the queue contains an event.

getEvent() — retrieves an event; exact behaviour when the queue is empty must be checked in EventGenerator.cpp.

clear() — clears queued events.

The supplied EventGenerator.h comments appear to have the comments for hasEvent() and getEvent() swapped. The function declarations are the reliable guide to their names; implementation details still need source verification.

# 7. Example Task Definition

The following illustrates how a developer can populate a Task. It is a template example, not a claim that these are the project's currently configured University requirements.

Task universityTask{};
universityTask.type = TaskType::University;
universityTask.requiredItems[0] = ItemType::Laptop;
universityTask.requiredItems[1] = ItemType::Phone;
universityTask.requiredItems[2] = ItemType::Wallet;
universityTask.itemCount = 3;
universityTask.reminderAudio = 1;

// After creating Backpack with a live Inventory instance:
bool registered = backpack.registerTask(universityTask);
if (registered) {
    bool started = backpack.startTask(TaskType::University);
}

# 8. Example Item Definition

The following illustrates how a developer can populate an Item. It is a template example on how to register items with the inventory, which sets its location flag to Location::Unknown internally.

inventory.registerItem(

RFIDTagID{{00, 00, 00, 01}, 4},

ItemType::Wallet

)

# 9. Typical Arduino Setup Pattern

The latest save point says a basic Arduino library sketch compiled on Uno, but its smoke-test example created Backpack locally inside setup(). That is only appropriate for a begin-only check. If loop() needs to use the objects, keep them alive for the lifetime of the sketch. The constructor in the supplied Backpack.h requires an Inventory reference.

#include <Backpack.h>

Inventory inventory;
Backpack backpack(inventory);

void setup() {
    Serial.begin(9600);
    backpack.begin();
    Serial.println("Assistive Backpack core ready");
}

void loop() {
    while (backpack.hasEvent()) {
        BackpackEvent event = backpack.getEvent();
        // Translate event.type / event.item / event.task into feedback.
    }
}

This snippet illustrates object lifetime and event consumption only. Add item registration, task setup, and RFID handling according to the current library API and hardware integration plan. Do not consume events in a debug console if another feedback component needs to receive the same events unless event dispatch is designed for it.

# 10. Inventory and Task Flow — Conceptual Example

Register each known RFID tag ID with an ItemType (for example, one tag maps to ItemType::Laptop).

Use the same Inventory instance for Backpack and TaskManager-related checks.

Register a Task with its TaskType, required item categories, itemCount, and reminderAudio value.

Start the task through Backpack::startTask(TaskType::University).

The core checks whether the task's required item categories are present in inventory.

The Backpack state may move through CHECKING_INVENTORY and then to ACQUIRING_ITEMS or TASK_REMINDER; confirm exact transitions in Backpack.cpp.

Report item entry/removal through the agreed Backpack hooks so the core can update inventory and task state.

Consume BackpackEvent values and map event types to feedback. The RFID scan alone does not inherently tell the system whether the item is entering or leaving.

# 11. Event Queue and Feedback Integration

The core and feedback layer are intentionally separate. The core reports what happened through BackpackEvent; the feedback layer decides how to communicate it. For example, ItemRequired might trigger a spoken prompt naming the required item, while TaskComplete might play a completion sound. These are integration examples; the actual audio mapping is not defined in the supplied headers.

EventGenerator has a fixed capacity of MAX_EVENTS (10). Since emit() returns bool, code should not assume every event was successfully queued. Decide which component owns event consumption. A single FIFO consumer removes events from the queue; multiple feedback consumers require an explicit dispatch/broadcast strategy.

# 12. Important Implementation Checks

Confirm the repository's current Inventory.h: the pasted header declares printInventory(), while the latest save point says it was removed from the shared core and getItemAt(index) was added.

Verify exact add/remove semantics and return values in Inventory.cpp.

Verify what getItemType(tagID) returns for an unregistered tag; use findItem() if the caller needs to distinguish missing from ItemType::Unknown.

Verify itemScanned(tagID) edge cases in Backpack.cpp, especially unknown tags and whether a scan can determine item entry versus exit.

Verify event-queue overflow and empty-queue behaviour in EventGenerator.cpp.

Use the exact case-sensitive event names from Events.h: UnknownRFID and ItemNotRequired.

Keep the shared core free of Arduino Serial, RC522 driver calls, audio drivers, and desktop iostream dependencies.

Build/test the desktop target and compile the Arduino sketch after changing shared headers.

Test capacity boundaries: 20 inventory records, 5 tasks, 10 required-item entries, and 10 queued events.

# 13. Source and Confidence Note

This guide is based on the header definitions supplied in the conversation and the latest available project save-point notes. The current Backpack.cpp, Inventory.cpp, TaskManager.cpp, and EventGenerator.cpp were not provided for this document-generation step, so implementation-specific edge cases and exact state transitions are identified as items to verify rather than presented as fully audited behaviour.